Wartens

info@wartens.com  ·  ladx.ai

# CPU 120 datasheet

LADV100 basic controller. Order number LV100-C120-20DDT-A3.

| Document | CPU 120 datasheet |
| --- | --- |
| Code | LADV-DS-120 |
| Revision | 1.0, 1 November 2026 |
| Firmware | V1.0.0 |
| Issued by | Wartens, makers of LADX |

Wartens LADV controllers are virtual PLCs that run in a web browser, for learning, for testing logic and for preparing for commissioning. They are not real time, not safety rated, and they cannot control equipment.

## Overview

Small fixed-I/O controllers for learning ladder logic. Enough inputs and outputs for a motor station, a set of traffic lights or a garage door, and small enough that every channel is in view.

12 digital in, 8 digital out, 2 analog in, 1 analog out.

## Technical data

| Property | Value |
| --- | --- |
| Series | LADV100 Basic |
| CPU | CPU 120 |
| Order number | LV100-C120-20DDT-A3 |
| Variant, as printed | DC/DC/DC |
| Digital inputs | 12 |
| Digital outputs | 8 |
| Analog inputs | 2 (INT, 0 to 1000 is 0 to 100 percent) |
| Analog outputs | 1 (INT, 0 to 1000 is 0 to 100 percent) |
| Expansion slots | None, fixed I/O |
| Program memory, ladder | 128 tags, 64 rungs, 4 routines |
| Program memory, Structured Text | 128 variables, 512 statements, 8 program organisation units |
| Fastest scan | 20 ms |
| Watchdog | 150 ms per scan |
| Retentive memory | Yes. A warm restart keeps RETAIN variables, and ladder memory |
| Languages | Ladder (LADX), IEC 61131-3 Structured Text |
| Tasks | Cyclic and event tasks, priorities from 0, non-preemptive |
| Online functions | Watch, modify, force I/O, 64-entry diagnostic buffer |
| Status lamps | RUN, STOP, ERROR, MAINT |
| Firmware | V1.0.0 |
| Serial number, as printed | WL26-120-50280 |
| Availability | Available in the VPLC lab |

## Addressing

Structured Text names a channel by its IEC 61131-3 direct address. Ladder wires tags to channels in the order the tags are listed.

| Channels | Count | Structured Text addresses |
| --- | --- | --- |
| Digital inputs | 12 | %IX0.0 to %IX1.3 |
| Digital outputs | 8 | %QX0.0 to %QX0.7 |
| Analog inputs | 2 | %IW0 to %IW1 |
| Analog outputs | 1 | %QW0 |

## Status lamps

| Lamp | Colour | Meaning |
| --- | --- | --- |
| RUN | Green | The program is executing and outputs are live. |
| STOP | Amber | Stopped. Outputs are off. |
| ERROR | Red | Faulted. Outputs are off until the fault is reset. The diagnostic buffer gives the code. |
| MAINT | Amber | At least one I/O point is forced. |

## Not included

- No fieldbus, no network protocols and no physical I/O.

- No hard real-time guarantee: scans run on simulated time in the browser.

- No safety rating. Safety functions belong in a safety-rated controller.

## Related documents

- LADV system manual, LADV-SM

- Structured Text programming manual, LADV-PM-ST

- Ladder instruction reference, LADV-IR-LD

- Diagnostic and fault codes, LADV-DC