Wartens

info@wartens.com  ·  ladx.ai

# CPU 350 datasheet

LADV300 advanced controller. Order number LV300-C350-16DDT.

| Document | CPU 350 datasheet |
| --- | --- |
| Code | LADV-DS-350 |
| Revision | 1.0, 1 November 2026 |
| Firmware | V1.0.0 |
| Issued by | Wartens, makers of LADX |

Wartens LADV controllers are virtual PLCs that run in a web browser, for learning, for testing logic and for preparing for commissioning. They are not real time, not safety rated, and they cannot control equipment.

## Overview

Modular controllers on a virtual backplane. Choose the CPU, slot in digital and analog modules, and size the system to the I/O list, the way a line controller is specified.

Modular CPU, 16 slots, the largest memory in the line.

## Technical data

| Property | Value |
| --- | --- |
| Series | LADV300 Advanced |
| CPU | CPU 350 |
| Order number | LV300-C350-16DDT |
| Variant, as printed | DC/DC/DC |
| Digital inputs | 8 |
| Digital outputs | 8 |
| Analog inputs | 0 |
| Analog outputs | 0 |
| Expansion slots | 16, for LADV-DI16, LADV-DO16, LADV-AI8, LADV-AO4 |
| Program memory, ladder | 8192 tags, 4096 rungs, 128 routines |
| Program memory, Structured Text | 8192 variables, 32768 statements, 256 program organisation units |
| Fastest scan | 5 ms |
| Watchdog | 150 ms per scan |
| Retentive memory | Yes. A warm restart keeps RETAIN variables, and ladder memory |
| Languages | Ladder (LADX), IEC 61131-3 Structured Text |
| Tasks | Cyclic and event tasks, priorities from 0, non-preemptive |
| Online functions | Watch, modify, force I/O, 64-entry diagnostic buffer |
| Status lamps | RUN, STOP, ERROR, MAINT |
| Firmware | V1.0.0 |
| Serial number, as printed | WL26-350-71650 |
| Availability | Available in the VPLC lab |

## Addressing

Structured Text names a channel by its IEC 61131-3 direct address. Ladder wires tags to channels in the order the tags are listed.

| Channels | Count | Structured Text addresses |
| --- | --- | --- |
| Digital inputs | 8 | %IX0.0 to %IX0.7 |
| Digital outputs | 8 | %QX0.0 to %QX0.7 |
| Analog inputs | 0 | none |
| Analog outputs | 0 | none |

Channels on modules follow the CPU's own, in slot order: a LADV-DI16 in the first slot after a CPU with 16 digital inputs gives DI16 to DI31, %IX2.0 to %IX3.7.

## Status lamps

| Lamp | Colour | Meaning |
| --- | --- | --- |
| RUN | Green | The program is executing and outputs are live. |
| STOP | Amber | Stopped. Outputs are off. |
| ERROR | Red | Faulted. Outputs are off until the fault is reset. The diagnostic buffer gives the code. |
| MAINT | Amber | At least one I/O point is forced. |

## Not included

- No fieldbus, no network protocols and no physical I/O.

- No hard real-time guarantee: scans run on simulated time in the browser.

- No safety rating. Safety functions belong in a safety-rated controller.

## Related documents

- LADV system manual, LADV-SM

- Structured Text programming manual, LADV-PM-ST

- Ladder instruction reference, LADV-IR-LD

- Diagnostic and fault codes, LADV-DC