Wartens

info@wartens.com  ·  ladx.ai

# Structured Text programming manual

IEC 61131-3 Structured Text on the Wartens LADV controllers: how a program is laid out, the data types, the statements, tasks and I/O, with listings that compile as printed.

| Document | Structured Text programming manual |
| --- | --- |
| Code | LADV-PM-ST |
| Revision | 1.0, 1 November 2026 |
| Firmware | V1.0.0 |
| Issued by | Wartens, makers of LADX |

Wartens LADV controllers are virtual PLCs that run in a web browser, for learning, for testing logic and for preparing for commissioning. They are not real time, not safety rated, and they cannot control equipment.

## 1. A first program

A Structured Text source holds declarations and program organisation units (POUs) in any order. The smallest useful program declares its I/O at direct addresses and has one PROGRAM:

VAR_GLOBAL

Start_PB AT %IX0.0 : BOOL;

Stop_PB  AT %IX0.1 : BOOL; (* normally closed: TRUE when healthy *)

Motor    AT %QX0.0 : BOOL;

END_VAR

PROGRAM Main

Motor := (Start_PB OR Motor) AND Stop_PB;

END_PROGRAM

With no CONFIGURATION, every PROGRAM runs in one cyclic task at the program's scan time. Identifiers are not case sensitive. Comments are written (* like this *), /* like this */ or // to the end of the line.

## 2. Elementary data types

| Type | Size | Range |
| --- | --- | --- |
| SINT | 8 bits, signed | -128 to 127 |
| INT | 16 bits, signed | -32,768 to 32,767 |
| DINT | 32 bits, signed | -2,147,483,648 to 2,147,483,647 |
| LINT | 64 bits, signed | -9,223,372,036,854,775,808 to 9,223,372,036,854,775,807, exact to 53 bits |
| USINT | 8 bits, unsigned | 0 to 255 |
| UINT | 16 bits, unsigned | 0 to 65,535 |
| UDINT | 32 bits, unsigned | 0 to 4,294,967,295 |
| ULINT | 64 bits, unsigned | 0 to 18,446,744,073,709,551,615, exact to 53 bits |
| BYTE | 8 bits, bit string | 16#0 to 16#FF |
| WORD | 16 bits, bit string | 16#0 to 16#FFFF |
| DWORD | 32 bits, bit string | 16#0 to 16#FFFFFFFF |
| LWORD | 64 bits, bit string | 16#0 to 16#FFFFFFFFFFFFFFFF, exact to 53 bits |

| Type | Meaning |
| --- | --- |
| BOOL | TRUE or FALSE |
| REAL, LREAL | IEC 60559 single and double precision floating point |
| TIME, LTIME | Durations: T#1h2m3s, T#250ms, LTIME#1.5us |
| DATE, TOD, DT | Dates and times of day: D#2026-11-01, TOD#08:30:00, DT#2026-11-01-08:30:00 |
| STRING, WSTRING | Text, 80 characters unless declared STRING[n] |
| CHAR, WCHAR | One character |

Integer literals can be written in base 2, 8 or 16 (2#1010, 8#17, 16#FF) and given a type (INT#5, BYTE#16#81). Integer arithmetic wraps at the width of the type, as hardware does: an INT holding 32767 plus 1 is -32768.

## 3. Derived types

Enumerations, subranges, arrays and structures are declared in a TYPE block. Structures and arrays can be nested and initialised.

TYPE

Mode : (Idle, Filling, Draining);

Percent : INT (0..1000);

Recipe : STRUCT

name : STRING[20] := 'Default';

setpoint : Percent := 600;

soak : TIME := T#30s;

END_STRUCT;

END_TYPE

PROGRAM Main

VAR

mode : Mode := Idle;

recipes : ARRAY[1..3] OF Recipe;

active : INT := 1;

target : Percent;

END_VAR

target := recipes[active].setpoint;

IF mode = Idle AND target > 0 THEN

mode := Filling;

END_IF;

END_PROGRAM

## 4. Variables

| Section | Where | Meaning |
| --- | --- | --- |
| VAR | Any POU | Local memory, kept between calls of a PROGRAM or FUNCTION_BLOCK |
| VAR_INPUT | FUNCTION, FUNCTION_BLOCK | Passed in by value |
| VAR_OUTPUT | FUNCTION_BLOCK | Read after the call as instance.name, or with => |
| VAR_IN_OUT | FUNCTION, FUNCTION_BLOCK | Passed by reference |
| VAR_TEMP | Any POU | Scratch memory, initialised on every call |
| VAR_GLOBAL | Outside POUs | Global memory and I/O, visible everywhere |

RETAIN keeps a variable through a warm restart on a retentive CPU. CONSTANT makes it read-only. AT gives a direct address: %IX and %QX for digital channels, %IW and %QW for analog, %M for internal memory.

## 5. Expressions

Operators from highest to lowest precedence: parentheses and calls; ** (power); unary - and NOT; * / MOD; + -; < > <= >=; = <>; AND and &; XOR; OR.

Implicit conversion only widens (INT to DINT, REAL to LREAL). Narrowing is a warning and needs a conversion function such as DINT_TO_INT to be explicit. A REAL is never stored in an integer without REAL_TO_INT or TRUNC. Comparing REAL values for equality is a warning, since rounding makes it unreliable.

## 6. Statements

| Statement | Form |
| --- | --- |
| Assignment | x := expression; |
| IF | IF c THEN ... ELSIF c THEN ... ELSE ... END_IF; |
| CASE | CASE n OF 1: ...; 2, 3: ...; 10..20: ...; ELSE ... END_CASE; |
| FOR | FOR i := 1 TO 10 BY 1 DO ... END_FOR; |
| WHILE | WHILE c DO ... END_WHILE; |
| REPEAT | REPEAT ... UNTIL c END_REPEAT; |
| EXIT, CONTINUE | Leave a loop, or start its next pass |
| RETURN | Leave the POU |
| Block call | timer(IN := start, PT := T#5s); or with outputs: fb(a := 1, q => result); |

## 7. Functions and function blocks

A FUNCTION returns a value and keeps no memory. A FUNCTION_BLOCK keeps its memory between calls and is used through an instance. Recursion is refused at compile time, and calls nest at most 32 deep.

FUNCTION Scale : REAL

VAR_INPUT

raw : INT;

lo, hi : REAL;

END_VAR

Scale := lo + (hi - lo) * INT_TO_REAL(raw) / 1000.0;

END_FUNCTION

FUNCTION_BLOCK Debounce

VAR_INPUT

raw : BOOL;

settle : TIME := T#50ms;

END_VAR

VAR_OUTPUT

clean : BOOL;

END_VAR

VAR

on_delay : TON;

off_delay : TON;

END_VAR

on_delay(IN := raw, PT := settle);

off_delay(IN := NOT raw, PT := settle);

IF on_delay.Q THEN

clean := TRUE;

ELSIF off_delay.Q THEN

clean := FALSE;

END_IF;

END_FUNCTION_BLOCK

PROGRAM Main

VAR

pressure : REAL;

door : Debounce;

door_shut : BOOL;

raw_pressure : INT := 500;

door_switch : BOOL;

END_VAR

pressure := Scale(raw := raw_pressure, lo := 0.0, hi := 10.0);

door(raw := door_switch, clean => door_shut);

END_PROGRAM

The standard functions and function blocks, from timers to string handling, are listed in LADV-IR-ST.

## 8. Tasks

A CONFIGURATION assigns programs to tasks. A cyclic task has an INTERVAL; an event task has SINGLE, and runs once on each rising edge of a global BOOL. At each scan the tasks that are due run in priority order, 0 first. Tasks do not preempt one another. A task faster than the CPU's fastest scan is refused at download.

VAR_GLOBAL

Batch_Start AT %IX0.0 : BOOL;

Heater      AT %QX0.0 : BOOL;

Temp        AT %IW0   : INT;

ticks : UDINT;

END_VAR

PROGRAM Control

Heater := Temp < 650;

END_PROGRAM

PROGRAM Housekeeping

ticks := ticks + 1;

END_PROGRAM

PROGRAM OnStart

VAR RETAIN

batches : UDINT;

END_VAR

batches := batches + 1;

END_PROGRAM

CONFIGURATION Cell

RESOURCE Cpu ON LADV

TASK Fast (INTERVAL := T#20ms, PRIORITY := 1);

TASK Slow (INTERVAL := T#500ms, PRIORITY := 5);

TASK Event (SINGLE := Batch_Start, PRIORITY := 0);

PROGRAM Loop WITH Fast : Control;

PROGRAM Stats WITH Slow : Housekeeping;

PROGRAM Count WITH Event : OnStart;

END_RESOURCE

END_CONFIGURATION

## 9. System variables

| Name | Meaning |
| --- | --- |
| FirstScan | TRUE during the first scan after a start, FALSE after |
| AlwaysTRUE, AlwaysFALSE | Constants |
| Clock_10Hz, Clock_5Hz, Clock_2Hz, Clock_1Hz, Clock_0_5Hz | Square waves at those frequencies |

## 10. Runtime errors

An error the compiler cannot see faults the controller, with a code and the line:

| Code | Cause |
| --- | --- |
| F300 | Watchdog: the scan took too long, or a loop never ended |
| F310 | Integer division or MOD by zero |
| F311 | Array index outside the declared range |
| F312 | MUX selector outside its inputs |
| F313 | BCD_TO_INT given a value that is not BCD |
| F314 | Calls nested more than 32 deep |

## 11. Not supported

- Object-oriented extensions: CLASS, METHOD, INTERFACE, PROPERTY.

- References and pointers: REF_TO, ^, POINTER TO.

- SFC elements in Structured Text source: STEP, TRANSITION, ACTION.

- NAMESPACE, and vendor libraries and system functions.

Each of these is refused with a message that names it, not a parse error.

## 12. Worked examples

The reference solutions for the lab's twins. Each passes every objective of its twin.

### Motor start/stop station

Start runs the motor and it keeps running when Start is released. Stop and E-Stop both stop it, and while E-Stop is pressed Start must do nothing. Light the Running lamp whenever the motor runs. Both stop buttons are wired normally closed, so they read 1 when nobody is pressing them.

(* Motor start/stop station: reference solution in IEC 61131-3 Structured Text. *)

VAR_GLOBAL

Start_PB AT %IX0.0  : BOOL; (* Start pushbutton, normally open *)

Stop_PB  AT %IX0.1  : BOOL; (* Stop pushbutton, normally closed, 1 when healthy *)

EStop_OK AT %IX0.2  : BOOL; (* Emergency stop, normally closed, 1 when healthy *)

Motor    AT %QX0.0  : BOOL; (* Motor contactor *)

Run_Lamp AT %QX0.1  : BOOL; (* Running indicator *)

END_VAR

PROGRAM Main

(* Start with a seal-in. Stop_PB and EStop_OK are normally closed, *)

(* TRUE while healthy, so a broken wire stops the motor too.       *)

Motor := (Start_PB OR Motor) AND Stop_PB AND EStop_OK;

Run_Lamp := Motor;

END_PROGRAM

### Traffic light junction

Run the junction: north-south green, then amber, then red while east-west goes green, amber, red, and round again. Never show two greens, always show amber between green and red, never leave a head dark, and give each direction at least four seconds of green.

(* Traffic light junction: reference solution in IEC 61131-3 Structured Text. *)

VAR_GLOBAL

NS_Red   AT %QX0.0  : BOOL; (* North-south red lamp *)

NS_Amber AT %QX0.1  : BOOL; (* North-south amber lamp *)

NS_Green AT %QX0.2  : BOOL; (* North-south green lamp *)

EW_Red   AT %QX0.3  : BOOL; (* East-west red lamp *)

EW_Amber AT %QX0.4  : BOOL; (* East-west amber lamp *)

EW_Green AT %QX0.5  : BOOL; (* East-west green lamp *)

END_VAR

TYPE Stage : (NS_GO, NS_WAIT, EW_GO, EW_WAIT); END_TYPE

PROGRAM Main

VAR

stage : Stage := NS_GO;

timer : TON;

hold : TIME;

END_VAR

(* How long each phase lasts. *)

CASE stage OF

NS_GO, EW_GO: hold := T#5s;

NS_WAIT, EW_WAIT: hold := T#2s;

END_CASE;

timer(IN := TRUE, PT := hold);

IF timer.Q THEN

timer(IN := FALSE);

CASE stage OF

NS_GO: stage := NS_WAIT;

NS_WAIT: stage := EW_GO;

EW_GO: stage := EW_WAIT;

EW_WAIT: stage := NS_GO;

END_CASE;

END_IF;

(* One lamp per head, decided only by the stage, so two greens cannot happen. *)

NS_Green := stage = NS_GO;

NS_Amber := stage = NS_WAIT;

NS_Red := stage = EW_GO OR stage = EW_WAIT;

EW_Green := stage = EW_GO;

EW_Amber := stage = EW_WAIT;

EW_Red := stage = NS_GO OR stage = NS_WAIT;

END_PROGRAM

### Sorting conveyor

Start and Stop run the belt, with a seal-in. Parts arrive on their own. At the pusher station a photo-eye sees every part and an inductive sensor sees only metal ones: push the metal parts into the reject bin and let the plastic parts ride to the end. Ten in a row with no mistakes passes.

(* Sorting conveyor: reference solution in IEC 61131-3 Structured Text. *)

VAR_GLOBAL

Start_PB     AT %IX0.0  : BOOL; (* Start pushbutton, normally open *)

Stop_PB      AT %IX0.1  : BOOL; (* Stop pushbutton, normally closed, 1 when healthy *)

PE_Entry     AT %IX0.2  : BOOL; (* Photo-eye at the feeder, 1 when a part is there *)

PE_Pusher    AT %IX0.3  : BOOL; (* Photo-eye at the pusher station, sees every part *)

Metal_Sensor AT %IX0.4  : BOOL; (* Inductive sensor at the pusher station, sees metal parts only *)

Belt         AT %QX0.0  : BOOL; (* Conveyor motor contactor *)

Pusher       AT %QX0.1  : BOOL; (* Reject pusher solenoid valve, 1 extends *)

END_VAR

PROGRAM Main

VAR

arrived : R_TRIG;

parts : UDINT; (* Parts seen at the entry since the last cold restart. *)

END_VAR

Belt := (Start_PB OR Belt) AND Stop_PB;

(* A metal part at the pusher station is pushed off. *)

Pusher := PE_Pusher AND Metal_Sensor;

arrived(CLK := PE_Entry);

IF arrived.Q THEN

parts := parts + 1;

END_IF;

END_PROGRAM

### Tank level control

The process draws water all the time. Open the fill valve when the level falls below 40 percent and close it above 80, so it never overflows and never runs dry. Copy the level to the panel meter. When the Auto selector is switched off, close the valve and keep it closed. Level reads 0 to 1000 for 0 to 100 percent.

(* Tank level control: reference solution in IEC 61131-3 Structured Text. *)

VAR_GLOBAL

Auto_Mode   AT %IX0.0  : BOOL; (* Auto/manual selector, 1 is auto *)

Level       AT %IW0    : INT; (* Level transmitter, 0 to 1000 is 0 to 100 percent *)

LSH         AT %IX0.1  : BOOL; (* High level switch, 1 above 90 percent *)

LSL         AT %IX0.2  : BOOL; (* Low level switch, 1 below 10 percent *)

Fill_Valve  AT %QX0.0  : BOOL; (* Fill valve solenoid, 1 opens *)

High_Alarm  AT %QX0.1  : BOOL; (* High level alarm lamp *)

Level_Meter AT %QW0    : INT; (* Panel meter, 0 to 1000 *)

END_VAR

PROGRAM Main

(* Level reads 0 to 1000 for 0 to 100 percent. Fill from 40 percent up to 80, *)

(* and never past the high switch or out of auto.                          *)

IF Auto_Mode AND Level < 400 THEN

Fill_Valve := TRUE;

END_IF;

IF Level > 800 OR LSH OR NOT Auto_Mode THEN

Fill_Valve := FALSE;

END_IF;

Level_Meter := LIMIT(0, Level, 1000);

High_Alarm := LSH;

END_PROGRAM