Wartens

info@wartens.com  ·  ladx.ai

# Structured Text standard library reference

Every standard function and function block the LADV controllers provide.

| Document | Structured Text standard library reference |
| --- | --- |
| Code | LADV-IR-ST |
| Revision | 1.0, 1 November 2026 |
| Firmware | V1.0.0 |
| Issued by | Wartens, makers of LADX |

## 1. Functions

Functions take inputs by position or by name and return one value. Those marked with ... take two or more inputs.

### Numeric

| Function | Call | Result |
| --- | --- | --- |
| ABS | ABS(IN) | Absolute value. |
| SQRT | SQRT(IN) | Square root of a REAL or LREAL. |
| LN | LN(IN) | Natural logarithm. |
| LOG | LOG(IN) | Logarithm to base 10. |
| EXP | EXP(IN) | e raised to IN. |
| SIN | SIN(IN) | Sine, IN in radians. |
| COS | COS(IN) | Cosine, IN in radians. |
| TAN | TAN(IN) | Tangent, IN in radians. |
| ASIN | ASIN(IN) | Arc sine, in radians. |
| ACOS | ACOS(IN) | Arc cosine, in radians. |
| ATAN | ATAN(IN) | Arc tangent, in radians. |
| ATAN2 | ATAN2(Y, X) | Angle of the point (X, Y) from the X axis, in radians. |
| EXPT | EXPT(IN1, IN2) | IN1 raised to the power IN2. |

### Arithmetic

| Function | Call | Result |
| --- | --- | --- |
| ADD | ADD(IN1, IN2, ...) | Sum. Same as +. |
| MUL | MUL(IN1, IN2, ...) | Product. Same as *. |
| SUB | SUB(IN1, IN2) | Difference. Same as -. |
| DIV | DIV(IN1, IN2) | Quotient. Same as /. Integer division truncates toward zero; by zero faults with F310. |
| MOD | MOD(IN1, IN2) | Remainder, with the sign of IN1. By zero faults with F310. |
| MOVE | MOVE(IN) | Returns IN. Same as assignment. |

### Bit shift

| Function | Call | Result |
| --- | --- | --- |
| SHL | SHL(IN, N) | Shift left N bits at the width of IN, zeros in. |
| SHR | SHR(IN, N) | Shift right N bits at the width of IN, zeros in. |
| ROL | ROL(IN, N) | Rotate left N bits at the width of IN. |
| ROR | ROR(IN, N) | Rotate right N bits at the width of IN. |

### Selection

| Function | Call | Result |
| --- | --- | --- |
| SEL | SEL(G, IN0, IN1) | IN1 when G is TRUE, otherwise IN0. |
| MAX | MAX(IN1, IN2, ...) | The largest input. |
| MIN | MIN(IN1, IN2, ...) | The smallest input. |
| LIMIT | LIMIT(MN, IN, MX) | IN held between MN and MX. |
| MUX | MUX(K, IN0, IN1, ...) | Input number K, counting from 0. K outside the inputs faults with F312. |

### Comparison

| Function | Call | Result |
| --- | --- | --- |
| GT | GT(IN1, IN2) | IN1 > IN2. |
| GE | GE(IN1, IN2) | IN1 >= IN2. |
| EQ | EQ(IN1, IN2) | IN1 = IN2. |
| LE | LE(IN1, IN2) | IN1 <= IN2. |
| LT | LT(IN1, IN2) | IN1 < IN2. |
| NE | NE(IN1, IN2) | IN1 <> IN2. |

### String

| Function | Call | Result |
| --- | --- | --- |
| LEN | LEN(IN) | Number of characters. |
| LEFT | LEFT(IN, L) | The first L characters. |
| RIGHT | RIGHT(IN, L) | The last L characters. |
| MID | MID(IN, L, P) | L characters starting at position P, counting from 1. |
| CONCAT | CONCAT(IN1, IN2, ...) | The inputs joined end to end. |
| INSERT | INSERT(IN1, IN2, P) | IN2 inserted into IN1 after position P. |
| DELETE | DELETE(IN, L, P) | IN with L characters removed from position P. |
| REPLACE | REPLACE(IN1, IN2, L, P) | L characters of IN1 from position P replaced by IN2. |
| FIND | FIND(IN1, IN2) | Position of the first IN2 in IN1, or 0 when it is not there. |

### Conversion

| Function | Call | Result |
| --- | --- | --- |
| TRUNC | TRUNC(IN) | REAL to integer, toward zero. |
| BCD_TO_INT | BCD_TO_INT(IN) | Binary-coded decimal to integer. A digit above 9 faults with F313. |
| INT_TO_BCD | INT_TO_BCD(IN) | Integer to binary-coded decimal. |

### Time and date

| Function | Call | Result |
| --- | --- | --- |
| ADD_TIME | ADD_TIME(IN1, IN2) | TIME + TIME. |
| SUB_TIME | SUB_TIME(IN1, IN2) | TIME - TIME. |
| MUL_TIME | MUL_TIME(IN1, IN2) | TIME multiplied by a number. |
| DIV_TIME | DIV_TIME(IN1, IN2) | TIME divided by a number. |
| ADD_TOD_TIME | ADD_TOD_TIME(IN1, IN2) | Time of day + TIME. |
| ADD_DT_TIME | ADD_DT_TIME(IN1, IN2) | Date and time + TIME. |
| SUB_TOD_TIME | SUB_TOD_TIME(IN1, IN2) | Time of day - TIME. |
| SUB_DT_TIME | SUB_DT_TIME(IN1, IN2) | Date and time - TIME. |
| SUB_DATE_DATE | SUB_DATE_DATE(IN1, IN2) | DATE - DATE, as TIME. |
| SUB_TOD_TOD | SUB_TOD_TOD(IN1, IN2) | Time of day - time of day, as TIME. |
| SUB_DT_DT | SUB_DT_DT(IN1, IN2) | Date and time - date and time, as TIME. |
| CONCAT_DATE_TOD | CONCAT_DATE_TOD(IN1, IN2) | A DATE and a time of day as one DT. |

### Type conversion

Any elementary type converts to another with X_TO_Y, for example INT_TO_REAL, REAL_TO_INT, DINT_TO_INT, BOOL_TO_INT, INT_TO_STRING, STRING_TO_INT, TIME_TO_DINT. TO_Y converts from whatever type the argument has. REAL to integer rounds half to even; TRUNC and REAL_TRUNC_INT truncate. An integer conversion that does not fit wraps at the width of the target, as hardware does.

## 2. Function blocks

Declare an instance, call it, and read its outputs as instance.name.

| Block | Inputs | Outputs | Behaviour |
| --- | --- | --- | --- |
| TON | IN : BOOL, PT : TIME | Q : BOOL, ET : TIME | On delay. Q goes TRUE when IN has been TRUE for PT. ET counts up to PT. IN FALSE resets both. |
| TOF | IN : BOOL, PT : TIME | Q : BOOL, ET : TIME | Off delay. Q follows IN up at once, and stays TRUE for PT after IN goes FALSE. |
| TP | IN : BOOL, PT : TIME | Q : BOOL, ET : TIME | Pulse. A rising edge on IN gives Q TRUE for exactly PT; IN cannot cut it short or extend it. |
| LTON | IN : BOOL, PT : LTIME | Q : BOOL, ET : LTIME | TON with LTIME. |
| LTOF | IN : BOOL, PT : LTIME | Q : BOOL, ET : LTIME | TOF with LTIME. |
| LTP | IN : BOOL, PT : LTIME | Q : BOOL, ET : LTIME | TP with LTIME. |
| CTU | CU : BOOL, R : BOOL, PV : INT | Q : BOOL, CV : INT | Count up. Each rising edge on CU adds one to CV. Q is CV >= PV. R sets CV to 0. |
| CTD | CD : BOOL, LD : BOOL, PV : INT | Q : BOOL, CV : INT | Count down. Each rising edge on CD takes one from CV. Q is CV <= 0. LD loads PV into CV. |
| CTUD | CU : BOOL, CD : BOOL, R : BOOL, LD : BOOL, PV : INT | QU : BOOL, QD : BOOL, CV : INT | Count up and down. QU is CV >= PV, QD is CV <= 0. R resets, LD loads PV. |
| CTU_DINT | CU : BOOL, R : BOOL, PV : DINT | Q : BOOL, CV : DINT | CTU with DINT counts. |
| CTD_DINT | CD : BOOL, LD : BOOL, PV : DINT | Q : BOOL, CV : DINT | CTD with DINT counts. |
| CTUD_DINT | CU : BOOL, CD : BOOL, R : BOOL, LD : BOOL, PV : DINT | QU : BOOL, QD : BOOL, CV : DINT | CTUD with DINT counts. |
| CTU_LINT | CU : BOOL, R : BOOL, PV : LINT | Q : BOOL, CV : LINT | CTU with LINT counts. |
| CTD_LINT | CD : BOOL, LD : BOOL, PV : LINT | Q : BOOL, CV : LINT | CTD with LINT counts. |
| CTUD_LINT | CU : BOOL, CD : BOOL, R : BOOL, LD : BOOL, PV : LINT | QU : BOOL, QD : BOOL, CV : LINT | CTUD with LINT counts. |
| CTU_UDINT | CU : BOOL, R : BOOL, PV : UDINT | Q : BOOL, CV : UDINT | CTU with UDINT counts. |
| CTD_UDINT | CD : BOOL, LD : BOOL, PV : UDINT | Q : BOOL, CV : UDINT | CTD with UDINT counts. |
| CTUD_UDINT | CU : BOOL, CD : BOOL, R : BOOL, LD : BOOL, PV : UDINT | QU : BOOL, QD : BOOL, CV : UDINT | CTUD with UDINT counts. |
| CTU_ULINT | CU : BOOL, R : BOOL, PV : ULINT | Q : BOOL, CV : ULINT | CTU with ULINT counts. |
| CTD_ULINT | CD : BOOL, LD : BOOL, PV : ULINT | Q : BOOL, CV : ULINT | CTD with ULINT counts. |
| CTUD_ULINT | CU : BOOL, CD : BOOL, R : BOOL, LD : BOOL, PV : ULINT | QU : BOOL, QD : BOOL, CV : ULINT | CTUD with ULINT counts. |
| R_TRIG | CLK : BOOL | Q : BOOL | Rising edge. Q is TRUE for one call when CLK goes from FALSE to TRUE. |
| F_TRIG | CLK : BOOL | Q : BOOL | Falling edge. Q is TRUE for one call when CLK goes from TRUE to FALSE. Not on the first call. |
| SR | S1 : BOOL, R : BOOL | Q1 : BOOL | Set dominant bistable. Q1 is TRUE after S1, FALSE after R, and TRUE when both are. |
| RS | S : BOOL, R1 : BOOL | Q1 : BOOL | Reset dominant bistable. Q1 is TRUE after S, FALSE after R1, and FALSE when both are. |

Timers measure time between calls, so a timer called every 10 ms has 10 ms resolution. Counters count edges, not scans: a CU held TRUE counts once.